| Contribution: | ESCO0725 |
| --- | --- |
| Date: | October 13, 2022 |
| Working Group: | Emergency Services (9-1-1) Working Group (ESWG) |
| Title: | Wireless Characteristics and Description of Formats for all Call Types |
| Issues Addressed: | Working document as we transition to NG9-1-1 |
| Source: | Erich Baumgartner / Siv Mohanraj Combined TIFs 81-82 Owners Emails: Erich.Baumgartner@rci.rogers.com mohanraj.sivagnanasundaram@bell.ca |
| Distribution: | ESTF0081 and ESTF0082; ESWG website |
| X-References: | TD 2015-531, TRP 2017-182 & related proceedings, TD 2018-188, TRP 2019-66, TD 2019-353, and TD 2021-199; plus ESTF0080 and all related tasks i.e. TIFs 81, 82, 83, 85, 86, 88, 89, 90, 91, 92, 93, and 94 |
| File Name: | ESCO0725_TIFs 81-82_Working Document - Wireless Characteristics and Description of formats |
| Notice: | This contribution has been prepared as basis for discussion in order to assist the Emergency Services Working Group. This contribution should not be construed as a binding proposal on the authors. Specifically, the authors reserve the right to amend, modify or withdraw this contribution at any time. |

|  | Characteristics | Wireless E9-1-1 Phase I Service | Dialable Call-Back Number (CBN) | Wireless NG/E9-1-1 Ph II Location Service & ICLU | Mobile Sub # | CBN digits format presented to ILEC | CBN digits format presented to PSAP | Solutions for Phase II Location if not presented at PSAP | Solutions for real CBN if not presented at PSAP | Customer Initiated NG9-1-1 VoLTE Call | Customer Initiated NG9-1-1 VoLTE & RTT9-1-1 Call | NG9-1-1 VoLTE Callback | RTT9-1-1 + VoLTE Callback |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Uninitiated | Handset has been manufactured and has some carrier information (such as a PRL) but no telephone number. This state includes any GSM handset without a SIM card. | Yes | No | Yes | Not Available | 911+7 digit IMEI | 911+7 digit IMEI | Not Applicable | WSPs’ 7x24 PSAP Support Centre | Yes Wireless 9-1-1 Phase II available ICLU available | Yes Wireless 9-1-1 Phase II available ICLU available | No No dialable callback number 911+7 digit IMEI | No No dialable callback number 911+7 digit IMEI |
| Unregistered | A state where the handset has not passed registration, such as immediately after power-up. The subscriber may be valid or invalid. Calls other than 9-1-1 require the handset to first successfully pass registration. | Yes | No | Yes | CDN (NPA-NXX-XXXX) | 911+7 digit IMEI | 911+7 digit IMEI | Not Applicable | WSPs’ 7x24 PSAP Support Centre | Yes Wireless 9-1-1 Phase II available ICLU available | Yes Wireless 9-1-1 Phase II available ICLU available | No No dialable callback number 911+7 digit IMEI | No No dialable callback number 911+7 digit IMEI |
| Active | Normal operation (registered to network). Handset can place and receive calls. Handset is operating within its home network. Wireless Phase II Stage 1 E9-1-1 locates will normally be provided to handsets in this state. | Yes | Yes | Yes | CDN (NPA-NXX-XXXX) | CDN (NPA-NXX-XXXX) | CDN (NPA-NXX-XXXX) | Not Applicable | Not Applicable | Yes Wireless 9-1-1 Phase II available ICLU available | Yes Wireless 9-1-1 Phase II available ICLU available | Phase 0: PSTN Phase 1: Callback ESInet –> OSP Q4 2023 See Steps 1-7 Phase 2: disabling supplemental feature (TBD) | Phase 0: N/A Phase 1: Callback ESInet –> OSP Q4 2023 See Steps 1-7 Phase 2: disabling supplemental feature (TBD) |
| Canadian / Domestic Roamer | A handset with a 10-digit Canadian telephone number and a valid subscription. Handset is roaming onto another Canadian WSP’s network. | Yes | Yes | Yes | CDN (NPA-NXX-XXXX) | CDN (NPA-NXX-XXXX) | CDN (NPA-NXX-XXXX) | Not Applicable | Not Applicable | Yes Wireless 9-1-1 Phase II should be available - TBC ICLU NOT available | Yes Wireless 9-1-1 Phase II should be available - TBC ICLU NOT available | No Roaming TBD | No Roaming TBD |
| North American Roamer | A handset with a 10 or 11 digit non-Canadian North American Numbering Plan (NANP) telephone number and a valid subscription. Handset is roaming onto a Canadian WSP’s network. | Yes | Yes | Yes. | NON –CDN (NPA-NXX-XXXX) | NON –CDN (NPA-NXX-XXXX) | NON –CDN (NPA-NXX-XXXX) | Not Applicable | Not Applicable | Yes Wireless 9-1-1 Phase II should be available - TBC ICLU NOT available | Yes Wireless 9-1-1 Phase II should be available - TBC ICLU NOT available | No Roaming TBD | No Roaming TBD |
| International Roamer | A handset with a non-NANP telephone number and a valid subscription. Handset is roaming onto a Canadian WSP’s network. | Yes | Yes, if 10 digits are transmitted to 9-1-1 Service Provider, and include Country Code (PSAP needs to dial 011 prefix) | Yes | CC X-XXX-XXXX | CC X-XXX-XXXX | CC X-XXX-XXXX | Not Applicable | WSPs’ 7x24 PSAP Support Centre | Yes Wireless 9-1-1 Phase II should be available - TBC ICLU NOT available | Yes Wireless 9-1-1 Phase II should be available - TBC ICLU NOT available | No Roaming TBD | No Roaming TBD |
| International Roamer (cont’d) |  | Yes | No, if 10 digits are transmitted to 9-1-1 Service Provider and do not include Country Code | Yes | XXX-XXX-XXXX | XXX-XXX-XXXX | XXX-XXX-XXXX | Not Applicable | WSPs’ 7x24 PSAP Support Centre | Yes Wireless 9-1-1 Phase II availability TBC ICLU NOT available | Yes Wireless 9-1-1 Phase II availability TBC ICLU NOT available | No Roaming TBD | No Roaming TBD |
| International Roamer (cont’d) |  | Yes | No, if other than 10 digits are transmitted to 9-1-1 Service Provider. | No, if other than 10 digits are transmitted to 9-1-1 Service Provider. | >/<XXX-XXX-XXXX | >/<XXX-XXX-XXXX | NPA-911-XXXX | Call WSPs’ 7x24 PSAP Support Centre (to obtain location) | PSAP call to 9-1-1 Service Provider Support Centre (to obtain CBN) | Yes Wireless 9-1-1 Phase II availability TBC ICLU NOT available | Yes Wireless 9-1-1 Phase II availability TBC ICLU NOT available | No Roaming TBD | No Roaming TBD |
| Lapsed Pre-Paid Subscription | Suspended by the carrier. This stage includes prepaid phones with an account balance of $0.00. The telephone number is still valid, but the handset cannot place or receive calls. The handset can call 9-1-1 or the WSP’s customer service. | Yes | No Telephone number is valid however due to suspension; the handset cannot be called. Note: Some WSP may optionally allow call back. | Yes | CDN (NPA-NXX-XXX) | CDN (NPA-NXX-XXX) or 911 + 7 digit IMEI | CDN (NPA-NXX-XXX) or 911 + 7 digit IMEI | Not Applicable | If PSAP needs to call back handset, contact WSP’s 7x24 PSAP Support Centre for support Note: Some WSP may optionally allow call back. | Yes Wireless 9-1-1 Phase II available ICLU available | Yes Wireless 9-1-1 Phase II available ICLU available | No Telephone number is valid however due to lapsed subscription; the handset cannot be called. | No Telephone number is valid however due to lapsed subscription; the handset cannot be called. |
| Unsubscribed | Disconnected by the carrier. The telephone number has been recycled by the carrier (aged then reassigned to another handset) or the subscriber has ported the telephone number to another carrier. | Yes | No | Yes | CDN (NPA-NXX-XXX) Note: Number does not belong to caller hence incorrect CBN | 911+7 digit IMEI | 911+7 digit IMEI | Not Applicable | WSPs’ 7x24 PSAP Support Centre for additional Subscribers OLD information. Note: Availability depends on length of deactivation. | Yes Wireless 9-1-1 Phase II available ICLU available | Yes Wireless 9-1-1 Phase II available ICLU available | No No dialable callback number 911+7 digit IMEI | No No dialable callback number 911+7 digit IMEI |
| Suspended | Carrier has temporarily suspended the user. The profile is active in the system. Caller can only dial 9-1-1 and customer service. | Yes | No | Yes | CDN (NPA-NXX-XXX) Note: number may be reactivated in some cases of suspension | CDN (NPA-NXX-XXX) or 911 + 7 digit IMEI | CDN (NPA-NXX-XXX) or 911 + 7 digit IMEI | Not Applicable | WSPs’ 7x24 PSAP Support Centre for additional Subscribers information | Yes Wireless 9-1-1 Phase II available ICLU available | Yes Wireless 9-1-1 Phase II available ICLU available | No Telephone number is valid however due to suspension; the handset cannot be called. | No Telephone number is valid however due to suspension; the handset cannot be called. |